APPLICATION:

#define BLYNK_TEMPLATE_ID "TMPL3hIoYXhCn"

#define BLYNK_TEMPLATE_NAME "home automation"

#define BLYNK_AUTH_TOKEN "prXQuuNzxsBCtQedRrZD_r-8XMpZGxvI"

//#define BLYNK_PRINT Serial

#include <BlynkSimpleEsp8266.h>

// define the GPIO connected with Relays and switches

#define RelayPin1 5  //D1

#define RelayPin2 4  //D2

#define RelayPin3 14 //D5

#define RelayPin4 12 //D6

#define SwitchPin1 10  //SD3

#define SwitchPin2 0   //D3

#define SwitchPin3 13  //D7

#define SwitchPin4 3   //RX

#define wifiLed   16   //D0

#define VPIN_BUTTON_1    V1

#define VPIN_BUTTON_2    V2

#define VPIN_BUTTON_3    V3

#define VPIN_BUTTON_4    V4

int toggleState_1 = 1; //Define integer to remember the toggle state for relay 1

int toggleState_2 = 1; //Define integer to remember the toggle state for relay 2

int toggleState_3 = 1; //Define integer to remember the toggle state for relay 3

int toggleState_4 = 1; //Define integer to remember the toggle state for relay 4

int wifiFlag = 0;

#define AUTH "prXQuuNzxsBCtQedRrZD_r-8XMpZGxvI"                 // You should get Auth Token in the Blynk App.

#define WIFI_SSID "iot"             //Enter Wifi Name

#define WIFI_PASS "123456789"         //Enter wifi Password

BlynkTimer timer;

void relayOnOff(int relay){

switch(relay){

case 1:

if(toggleState_1 == 1){

digitalWrite(RelayPin1, LOW); // turn on relay 1

toggleState_1 = 0;

Serial.println("Device1 ON");

}

else{

digitalWrite(RelayPin1, HIGH); // turn off relay 1

toggleState_1 = 1;

Serial.println("Device1 OFF");

}

delay(100);

break;

case 2:

if(toggleState_2 == 1){

digitalWrite(RelayPin2, LOW); // turn on relay 2

toggleState_2 = 0;

Serial.println("Device2 ON");

}

else{

digitalWrite(RelayPin2, HIGH); // turn off relay 2

toggleState_2 = 1;

Serial.println("Device2 OFF");

}

delay(100);

break;

case 3:

if(toggleState_3 == 1){

digitalWrite(RelayPin3, LOW); // turn on relay 3

toggleState_3 = 0;

Serial.println("Device3 ON");

}

else{

digitalWrite(RelayPin3, HIGH); // turn off relay 3

toggleState_3 = 1;

Serial.println("Device3 OFF");

}

delay(100);

break;

case 4:

if(toggleState_4 == 1){

digitalWrite(RelayPin4, LOW); // turn on relay 4

toggleState_4 = 0;

Serial.println("Device4 ON");

}

else{

digitalWrite(RelayPin4, HIGH); // turn off relay 4

toggleState_4 = 1;

Serial.println("Device4 OFF");

}

delay(100);

break;

default : break;

}

}

void with_internet(){

//Manual Switch Control

if (digitalRead(SwitchPin1) == LOW){

delay(200);

relayOnOff(1);

Blynk.virtualWrite(VPIN_BUTTON_1, toggleState_1);   // Update Button Widget

}

else if (digitalRead(SwitchPin2) == LOW){

delay(200);

relayOnOff(2);

Blynk.virtualWrite(VPIN_BUTTON_2, toggleState_2);   // Update Button Widget

}

else if (digitalRead(SwitchPin3) == LOW){

delay(200);

relayOnOff(3);

Blynk.virtualWrite(VPIN_BUTTON_3, toggleState_3);   // Update Button Widget

}

else if (digitalRead(SwitchPin4) == LOW){

delay(200);

relayOnOff(4);

Blynk.virtualWrite(VPIN_BUTTON_4, toggleState_4);   // Update Button Widget

}

}

void without_internet(){

//Manual Switch Control

if (digitalRead(SwitchPin1) == LOW){

delay(200);

relayOnOff(1);

}

else if (digitalRead(SwitchPin2) == LOW){

delay(200);

relayOnOff(2);

}

else if (digitalRead(SwitchPin3) == LOW){

delay(200);

relayOnOff(3);

}

else if (digitalRead(SwitchPin4) == LOW){

delay(200);

relayOnOff(4);

}

}

BLYNK_CONNECTED() {

// Request the latest state from the server

Blynk.syncVirtual(VPIN_BUTTON_1);

Blynk.syncVirtual(VPIN_BUTTON_2);

Blynk.syncVirtual(VPIN_BUTTON_3);

Blynk.syncVirtual(VPIN_BUTTON_4);

}

// When App button is pushed - switch the state

BLYNK_WRITE(VPIN_BUTTON_1) {

toggleState_1 = param.asInt();

digitalWrite(RelayPin1, toggleState_1);

}

BLYNK_WRITE(VPIN_BUTTON_2) {

toggleState_2 = param.asInt();

digitalWrite(RelayPin2, toggleState_2);

}

BLYNK_WRITE(VPIN_BUTTON_3) {

toggleState_3 = param.asInt();

digitalWrite(RelayPin3, toggleState_3);

}

BLYNK_WRITE(VPIN_BUTTON_4) {

toggleState_4 = param.asInt();

digitalWrite(RelayPin4, toggleState_4);

}

void checkBlynkStatus() { // called every 3 seconds by SimpleTimer

bool isconnected = Blynk.connected();

if (isconnected == false) {

wifiFlag = 1;

digitalWrite(wifiLed, HIGH); //Turn off WiFi LED

}

if (isconnected == true) {

wifiFlag = 0;

digitalWrite(wifiLed, LOW); //Turn on WiFi LED

}

}

void setup()

{

Serial.begin(9600);

pinMode(RelayPin1, OUTPUT);

pinMode(RelayPin2, OUTPUT);

pinMode(RelayPin3, OUTPUT);

pinMode(RelayPin4, OUTPUT);

pinMode(wifiLed, OUTPUT);

pinMode(SwitchPin1, INPUT_PULLUP);

pinMode(SwitchPin2, INPUT_PULLUP);

pinMode(SwitchPin3, INPUT_PULLUP);

pinMode(SwitchPin4, INPUT_PULLUP);

//During Starting all Relays should TURN OFF

digitalWrite(RelayPin1, toggleState_1);

digitalWrite(RelayPin2, toggleState_2);

digitalWrite(RelayPin3, toggleState_3);

digitalWrite(RelayPin4, toggleState_4);

WiFi.begin(WIFI_SSID, WIFI_PASS);

timer.setInterval(3000L, checkBlynkStatus); // check if Blynk server is connected every 3 seconds

Blynk.config(AUTH);

}

void loop()

{

if (WiFi.status() != WL_CONNECTED)

{

Serial.println("WiFi Not Connected");

}

else

{

Serial.println("WiFi Connected");

Blynk.run();

}

timer.run(); // Initiates SimpleTimer

if (wifiFlag == 0)

with_internet();

else

without_internet();

}

WIFI

#include <ESP8266WiFi.h>

// Replace with your WiFi credentials

const char* ssid = "iot";            // Your Wi-Fi SSID

const char* password = "123456789";  // Your Wi-Fi Password

void setup() {

Serial.begin(9600); // Start the Serial communication

delay(100);

// Begin WiFi

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

// Connected

Serial.println();

Serial.println("WiFi connected successfully!");

Serial.print("IP address: ");

Serial.println(WiFi.localIP()); // Print local IP address

}

void loop() {

// Nothing to do here for now

}